SPECTRAL ANALYSIS FEATURE

Architecture & Design Document

Perceptiv Admin Dashboard

| Version | 1.0 |
| --- | --- |
| Author | Harsha |
| Date | February 2026 |
| Status | Draft |
| Phase | MVP / Phase 1 |

# 1. Executive Summary

This document defines the technical architecture for the Spectral Analysis feature within the Perceptiv Admin Dashboard. The feature enables vibration analysts to investigate raw waveform data and FFT spectra directly from alarm contexts, supporting root cause analysis of equipment health issues.

The design prioritizes a clean separation between a data-serving backend API and a client-side visualization layer. FFT computation is performed in the browser, keeping the backend as a reusable data service that can serve both the Perceptiv dashboard and potential third-party consumers.

# 2. Architectural Decisions

The following key decisions were made during design and are documented here with rationale for traceability.

| ID | Decision | Rationale |
| --- | --- | --- |
| ADR-001 | FFT computed client-side in the browser | Eliminates API round-trip on axis toggle. Enables backend to remain a pure data service. Browser can compute FFT on 16,384 points in <50ms using optimized JavaScript libraries. Third-party API consumers can perform their own analysis. |
| ADR-002 | Backend decodes base64 and scales to g values | Raw sensor data is base64-encoded 16-bit signed integers. Decoding and scaling require knowledge of config.accelRange_g. Centralizing this in the backend ensures consistent data interpretation across all consumers. |
| ADR-003 | Plotly.js for chart rendering | Provides zoom, pan, hover tooltips out of the box. Handles 16,384 data points with interactive performance. Supports scientific chart conventions (axis labels, units). Significantly reduces frontend development time compared to D3.js. |
| ADR-004 | All three axes loaded in a single API call | A single waveform payload contains rawHoriz, rawVert, and rawAxial. Loading all three at once avoids repeated API calls when the analyst toggles between axes. Payload size is ~100KB decoded per axis, totaling ~300KB — acceptable for desktop use. |
| ADR-005 | Sample rate read from sensor config metadata | Actual sample rates vary per sensor (e.g., 6547.3 Hz vs 6513.9 Hz). The system must use config.sampleRate_Hz from each payload, not hardcoded nominal values, to ensure accurate frequency axis computation. |

# 3. System Architecture

## 3.1 High-Level Data Flow

The spectral analysis feature follows a linear data pipeline from sensor hardware through to browser-rendered visualization.

| SR-71VT Sensor | Gateway | Azure Cloud (ADLS) | Backend API (Node.js) | Frontend (React) |
| --- | --- | --- | --- | --- |
| Collects raw acceleration at ~6547 Hz for ~2.5 seconds | Receives telemetry, forwards to cloud | Stores raw waveform as base64-encoded JSON blobs | Reads from ADLS, decodes base64, scales to g values, returns JSON arrays | Renders time waveform, computes FFT client-side, Plotly.js charts |

Data transformation at each stage: The sensor captures 16,384 samples (2^14, a power of 2 optimized for FFT) as raw acceleration data. The gateway transmits this as telemetry to Azure, where it is stored in ADLS as a JSON blob containing base64-encoded 16-bit signed integers. The backend API reads this blob, decodes the base64 string, unpacks the 16-bit integers, and scales them using the formula: value_g = raw_int × (accelRange_g / 32768). The frontend receives clean g-value arrays and computes FFT in the browser.

## 3.2 Backend API Design

### 3.2.1 API Endpoints

| Endpoint | Method | Purpose | Requirement |
| --- | --- | --- | --- |
| /api/waveform/{sensorId}/{timestamp} | GET | Returns decoded waveform data for all three axes at the specified timestamp. Reads raw blob from ADLS, decodes base64, scales to g values. | FR-001, FR-002, FR-006 |
| /api/waveform/{sensorId}/compare?t1={ts1}&t2={ts2} | GET | Returns decoded waveform data for two timestamps, enabling side-by-side spectral comparison. | FR-007 |

### 3.2.2 Response Schema – Single Waveform

The API returns a JSON object containing sensor metadata and decoded acceleration arrays for all three axes.

{

"sensorId": "12242400020011",

"timestamp": 1761254160,

"sensorType": "SR-71VT",

"config": {

"sampleRate_Hz": 6547.314578005115,

"accelRange_g": 16

},

"data": {

"horiz": [0.1074, 0.0503, 0.0317, ...],  // 16384 values in g

"vert":  [0.0892, 0.0411, 0.0285, ...],  // 16384 values in g

"axial": [0.0654, 0.0389, 0.0198, ...]   // 16384 values in g

},

"metadata": {

"sampleCount": 16384,

"duration_s": 2.5024,

"nyquist_Hz": 3273.66,

"frequencyResolution_Hz": 0.3996

}

}

### 3.2.3 Response Schema – Comparison

The comparison endpoint wraps two single waveform responses.

{

"sensorId": "12242400020011",

"baseline": { /* same structure as single waveform */ },

"current":  { /* same structure as single waveform */ }

}

### 3.2.4 Backend Processing Logic

The backend performs the following steps when a waveform request is received:

- Authenticate request using existing JWT token from the Perceptiv session.

- Query ADLS for the raw waveform blob matching the sensorId and timestamp.

- Decode base64 strings (rawHoriz, rawVert, rawAxial) into byte arrays.

- Unpack as 16-bit signed little-endian integers (Int16LE), yielding 16,384 integer values per axis.

- Scale to g values using: value_g = raw_int × (config.accelRange_g / 32768).

- Compute derived metadata: duration, Nyquist frequency, and frequency resolution from config.sampleRate_Hz and sample count.

- Return JSON response with all three axes and metadata.

## 3.3 Frontend Architecture

### 3.3.1 Component Hierarchy

| Component | Responsibility | Requirement |
| --- | --- | --- |
| SpectralAnalysisPage | Top-level page container. Manages API calls, holds waveform state for all three axes. Orchestrates child component layout. | FR-001 |
| AxisSelector | Three-button toggle (Horizontal / Vertical / Axial). Emits selected axis to parent. No page reload on switch. | FR-002 |
| TimeWaveformChart | Renders time-domain plot using Plotly.js. X-axis: time in seconds (0 to duration). Y-axis: acceleration in g. Supports zoom, pan, reset, hover tooltip. | FR-002, FR-004, FR-005 |
| FFTChart | Computes FFT from time-domain data using fft.js library. Renders frequency-domain plot. X-axis: frequency in Hz (0 to Nyquist). Y-axis: amplitude in g rms. Hover tooltip shows frequency, amplitude, and CPM. | FR-002, FR-003, FR-004, FR-005 |
| SpectralComparisonView | Fetches two timestamps, displays two FFT charts side-by-side for the same sensor and axis. | FR-007 |
| ErrorBoundary | Catches data loading failures. Displays user-friendly error message. Prevents partial/corrupted chart rendering. | AC-002.7 |

### 3.3.2 State Management

The component uses React state (useState/useReducer) to manage the following state variables:

| State Variable | Type | Purpose |
| --- | --- | --- |
| waveformData | { horiz: number[], vert: number[], axial: number[] } | Decoded acceleration arrays for all three axes, loaded once per timestamp |
| config | { sampleRate_Hz: number, accelRange_g: number } | Sensor configuration metadata from the API response |
| metadata | { sampleCount, duration_s, nyquist_Hz, frequencyResolution_Hz } | Computed metadata used for axis scaling |
| selectedAxis | 'horiz' ¦ 'vert' ¦ 'axial' | Currently active axis. Toggling this re-renders charts from already-loaded data (no API call) |
| isLoading | boolean | Controls loading spinner visibility during API fetch |
| error | string ¦ null | Error message to display if data fetch or processing fails |

### 3.3.3 FFT Computation (Client-Side)

The frontend computes FFT using the fft.js library, which is optimized for power-of-2 input sizes. The 16,384-sample input (2^14) is ideal for this.

FFT processing steps within the FFTChart component:

- Receive the time-domain g-values array for the selected axis from parent state.

- Apply a Hanning window to reduce spectral leakage (a common signal processing step that minimizes edge effects in FFT).

- Compute the FFT, yielding complex output (real and imaginary components).

- Calculate magnitude spectrum: magnitude = sqrt(real² + imag²) / N, where N is the sample count.

- Generate the frequency axis: each bin k corresponds to frequency = k × (sampleRate_Hz / N), from 0 Hz to Nyquist (sampleRate_Hz / 2).

- Render using Plotly.js with hover tooltip displaying: frequency (Hz), amplitude (g rms), and CPM (frequency × 60).

## 3.4 Page Layout

The spectral analysis view follows the layout pattern established by the existing Perceptiv Data Viewer, adapted for the admin dashboard context.

HEADER BAR: Sensor ID ¦ Timestamp ¦ Sensor Type ¦ Health Status

AXIS SELECTOR: [ Horizontal ] [ Vertical ] [ Axial ]

TIME WAVEFORM CHART X: Time (seconds, 0 to ~2.5s) ¦ Y: Acceleration (g) Interactive: Zoom, Pan, Reset, Hover Cursor

FFT SPECTRUM CHART X: Frequency (Hz, 0 to ~3274 Hz) ¦ Y: Amplitude (g rms) Interactive: Zoom, Pan, Reset, Hover Cursor with Hz + g rms + CPM

# 4. Data Specifications

## 4.1 Raw Waveform Data Profile

Based on analysis of actual SR-71VT sensor data captured from the production environment.

| Parameter | Value | Source / Derivation |
| --- | --- | --- |
| Sensor Type | SR-71VT | Payload field: sensorType |
| Sampling Rate | ~6547 Hz (varies per sensor) | Payload field: config.sampleRate_Hz |
| Accelerometer Range | ±16 g | Payload field: config.accelRange_g |
| Samples per Capture | 16,384 (2^14) | Byte count / 2 (16-bit integers) |
| Capture Duration | ~2.50 seconds | sampleCount / sampleRate_Hz |
| Nyquist Frequency | ~3274 Hz | sampleRate_Hz / 2 |
| FFT Frequency Resolution | ~0.40 Hz | sampleRate_Hz / sampleCount |
| FFT Lines (output bins) | 8,192 | sampleCount / 2 |
| Raw Encoding | Base64 → 16-bit signed LE integers | Verified by decoding rawHoriz |
| Scaling Formula | raw_int × (accelRange_g / 32768) | Standard ADC conversion |
| Amplitude Range (typical) | -0.12g to +0.12g | Measured from sample data |
| Axes Available | Horizontal, Vertical, Axial | Fields: rawHoriz, rawVert, rawAxial |
| Low Frequency Data | Not available (empty fields) | Fields: lowFreqHoriz/Vert/Axial = empty |

## 4.2 Calculated Data (Existing – from Sensor)

For context, these parameters are already computed by the SR-71VT sensor hardware and displayed in the existing Power BI dashboard. They represent the screening data that triggers investigation.

| Parameter | Unit (Metric) | Unit (Imperial) | Description |
| --- | --- | --- | --- |
| RMS | g | inches/sec (ips) | Root Mean Square – average vibration energy |
| Peak | g | ips | Maximum instantaneous amplitude |
| Peak-to-Peak | g | ips | Full swing: lowest valley to highest peak |
| Crest Factor | g/g (ratio) | g/g (ratio) | Peak ÷ RMS. Values >4 suggest early bearing damage |
| 1x Magnitude | ips | ips | Amplitude at shaft rotation frequency (imbalance indicator) |
| 2x Magnitude | ips | ips | Amplitude at 2× rotation frequency (misalignment indicator) |
| High Frequency | g | g | Energy in high-frequency band (bearing/gear mesh) |
| Temperature | °C | °F | Sensor surface temperature |
| Battery Voltage | V | V | Sensor battery level |

# 5. Technology Stack

| Layer | Technology | Purpose |
| --- | --- | --- |
| Frontend Framework | React 18 + TypeScript | Component-based UI with type safety |
| Charting Library | Plotly.js | Interactive scientific charts with zoom, pan, hover, tooltips |
| FFT Library | fft.js (or similar) | Client-side FFT computation optimized for power-of-2 inputs |
| Backend Runtime | Node.js + Express | API server for waveform data retrieval and decoding |
| Cloud Storage | Azure Data Lake Storage (ADLS) | Raw waveform blob storage |
| Database | Cosmos DB (Mongo API) | Unified message and calculated data storage |
| Authentication | JWT (existing Perceptiv auth) | Session-based access control |

# 6. Phased Delivery Plan

The feature is scoped for phased delivery to manage complexity and allow incremental stakeholder feedback.

| Phase | Scope | Key Deliverables |
| --- | --- | --- |
| Phase 1 (MVP) | Core spectral visualization for a single sensor, single timestamp | Time waveform chart, FFT spectrum chart, axis toggle (H/V/A), hover cursor with frequency + amplitude + CPM, zoom/pan/reset, error handling |
| Phase 2 | Enhanced analyst interaction | Dual cursor with delta measurement (time/frequency difference between two user-placed cursors), spectral comparison (side-by-side FFT for two timestamps) |
| Phase 3 | Machine context integration | Order calculation (requires RPM input or machine config integration), bearing defect frequency overlay, expanded tooltip with 1X/2X/order markers |
| Future | Advanced capabilities (not currently scoped) | Low-frequency waveform support (400 Hz), velocity conversion (integration of acceleration), automated peak detection and fault pattern recognition |

# 7. Non-Functional Requirements – Architecture Mapping

Each non-functional requirement is addressed by a specific architectural approach.

| NFR | Requirement | Architectural Approach |
| --- | --- | --- |
| NFR-001 | Render within 5 seconds (click to interactive chart) | Backend caching of recently accessed blobs. Single API call for all 3 axes. Client-side FFT avoids second round-trip. Plotly.js WebGL renderer for large datasets. |
| NFR-002 | 50 concurrent analysts without >10% degradation | Stateless backend API. Horizontal scaling via container orchestration. ADLS read operations are inherently parallelizable. |
| NFR-003 | Chrome, Edge, Firefox (latest 2 versions) | Plotly.js and fft.js are compatible with all modern browsers. No browser-specific APIs used. |
| NFR-004 | Exact metadata-based computation (no rounding) | Sample rate passed through API response as-is from source JSON. Frontend FFT uses the exact value for frequency axis generation. No intermediate rounding in the pipeline. |

# 8. Risks and Mitigations

| Risk | Impact | Likelihood | Mitigation |
| --- | --- | --- | --- |
| ADLS read latency exceeds 3 seconds | NFR-001 breach – total render time exceeds 5s target | Medium | Implement backend caching layer for recently accessed waveform blobs. Monitor P95 latency. |
| Browser memory constraints with multiple tabs | Analysts opening many spectral views simultaneously may cause browser slowdown | Low | Implement data cleanup on component unmount. Limit comparison to 2 timestamps. |
| Sensor sample rate variation affects FFT accuracy | Incorrect frequency axis if nominal rates are assumed | High (already mitigated) | ADR-005 ensures exact config.sampleRate_Hz is used throughout the pipeline. |
| Plotly.js performance at 16,384 points with zoom | Choppy interaction during rapid zoom/pan | Low | Enable Plotly WebGL renderer (scattergl trace type). Test with target browsers. |
| Future low-frequency data changes payload structure | Backend decoder breaks if field format differs | Medium | Design decoder to handle empty fields gracefully. Add payload version check using schemaVersion field. |

# 9. Out of Scope (Phase 1)

The following are explicitly excluded from Phase 1 to maintain focus and manage delivery risk.

- Alert threshold configuration or rule engine changes

- Automated report generation

- Historical comparison across multiple months

- Real-time waveform streaming

- Bearing fault frequency database integration

- Predictive maintenance recommendation engine

- Velocity conversion (requires time-domain integration of acceleration data)

- Low-frequency waveform visualization (400 Hz data not currently available)

- Mobile or tablet support (desktop only per NFR-003/NFR-004)

# 10. Glossary

| Term | Definition |
| --- | --- |
| FFT | Fast Fourier Transform – algorithm that converts a time-domain signal into its frequency components |
| Nyquist Frequency | Maximum observable frequency = sample rate ÷ 2. Frequencies above this cannot be detected. |
| Frequency Resolution | Smallest frequency difference distinguishable in the FFT = sample rate ÷ sample count |
| RMS | Root Mean Square – statistical measure of average signal energy |
| Crest Factor | Ratio of peak amplitude to RMS. Indicates impulsiveness of the signal. |
| CPM | Cycles Per Minute = frequency in Hz × 60. Common in rotating machinery analysis. |
| Order (1X, 2X) | Multiples of shaft rotation frequency. 1X = imbalance, 2X = misalignment. |
| BPFO / BPFI | Ball Pass Frequency Outer/Inner – bearing defect frequencies calculated from geometry |
| ADLS | Azure Data Lake Storage – cloud storage for large binary/JSON data |
| Hanning Window | Windowing function applied before FFT to reduce spectral leakage at signal edges |
| Spectral Leakage | Artifact in FFT caused by non-periodic signal edges, spreading energy across bins |
| Base64 | Encoding scheme that represents binary data as ASCII text for safe transmission in JSON |